Manufacturing Team Challenge

Shizouka Japan 2007

# Competition Information Pack

Contents								             Page

1. Project Brief 	2

2. Assessment Criteria	5

3. Competition Requirements	5

4. Activities to be done before the competition	5

5. Activities to be attempted during the competition 	5

6. Equipment/Items provided by the organizers	6

7. Items to be provided by the teams	6

8. Equipment and Materials not permitted	7

9. Surprise Project 	8

10. Main project - compliance to specification	9

  - Wind Generator	9

  - Pump	11

  - Light 	13

  - Filter 	14

  - Weight	15

  - Assembly	16

  - Transportation Enclosure 	17

  - Guarding 	18

  - Maintenance and Repair 	19

  - Drawings	20

  - Manuals 	22

  - Manufacturing Schedule	23

11. Material & Production costs 	24

12. Portfolio Assessment 	27

Release details:

IP03_39JP_01_EN version posted on MTC Forum on 15th February 2007

1. Project Brief

# Wind Generator, Water Pump and Water Filter.

# Japan unfortunately has many natural disasters, such as earthquakes and storms and once an earthquake or severe storm happens, all services such as electricity, water and gas stop immediately making people's life very difficult.

# This proposal is for an integral unit consisting of a wind generator, which would charge a battery. The battery would then be used to power a light and also a small water pump which would pump water to a filtering system.

# This unit should be portable and transported quickly to a disaster area and be quick to setup and provide some light and also some drinking water to affected people. Families or communities may keep a unit in their own homes in case of a severe disaster.

# The wind generator, must be portable, and be able to automatically change directions to suit the prevailing wind direction. All moving parts must be guarded to prevent any injury. The generator would charge a battery but only over a 12 hour period. The battery must be capable of illuminating a light and also to power the pump.

# The blades or turbine must be made during the competition but the electric generator or dynamo unit may be bought into the competition. All guards and supporting structure are to be made during the competition. The transport enclosure may be used as a mounting structure for the unit.

# The generator must have a visual indication of charging rate and also a visual indicator on the charged condition of the battery.

# The light must have a power of not less than 3 watts, and be capable of operating continuously for 12 hours from a single battery charge. The light must be able to operate at a distance of at least 5 metres from the generator unit.

# The pump must be capable of pumping 10 litres of fresh water per hour, and it will be considered that the water level is available at ground level.

# The pump may stop and start during pumping operation, and does not need to run constantly, but must be capable of pumping at least 10 litres per hour over 24 hours per day.

# The pump will pump the water to a filtering system.

# The complete pump would need to be made during the competition, but the electric motor unit may be bought into the competition.

# The filter system is to remove particles of dust and dirt and other contaminants from the water supply to a drinkable condition. The filter should be able to be easily cleaned or replaced, with an indicator to indicate when the filter flow is impeded by a buildup of contaminants. Water from the filter system is to be stored in a container of at least 20 litres.

# Transport Enclosure: The unit must be lightweight and easy to transport and fit within a transport enclosure of combined length, width and height of not greater than 1800mm (all maximum sizes to be added). The transport enclosure must be capable of withstanding a static load of 200N acting over an area of 75 mm diameter. This transport enclosure can be made before the competition, but will be included in the testing.

# The unit should be quick to remove from the transport enclosure and set up ready for operation, and also to pack up and to replace the unit into the transport enclosure. The enclosure may be used as a mounting structure or container for the unit.

# The battery must be disconnected when packing the unit into the transport container, and it will be considered that the battery will be transported in an upright position.

# Maintenance: The unit should be easily maintained and/or repaired with modular items requiring minimum skill and tooling requirements. Any tooling for repair and/or maintenance should be supplied and capable of being transported within the transport enclosure.

# The electrical/electronic circuit diagram is to be drawn in a computer format during the competition. A hand drawn sketch of the circuit may be bought into the competition.

# An assembly and manufacturing drawing of each of the unit components is to be drawn in a computer format during the competition. Hand drawn sketches of this assembly may be bought into the competition.

# Operating instructions for the unit are to be prepared during the competition and include directions for cleaning or changing the filter. The light must have a replacement globe and the globe must be able to be changed quickly.

# A portfolio is to be presented prior to the competition. This portfolio is to be assessed as a component this project. This portfolio to be in electronic format but presented in both hard copy and electronic form and to include:

# team member details

# team preparation

# design calculations and sketches

# lists of materials and components used with catalogue prices

# evidence of material and component costs

# computer generated 3 day competition manufacturing schedule

# 2D drawings of all manufactured parts

# 3D model drawing

# Teams will present a work schedule by the end of the first day of the competition, showing a revised schedule of their planned activities, men and machines for the duration of the competition and to include the work involved with the surprise project. This schedule is to be done with electronic project management software.

# A poster display 1000mm x 600mm (minimum size) detailing each team member’s details and their employers, a description of the project solution including a 3D model of their unit, which will be presented prior to the competition and displayed during the competition.

# 2. Assessment Criteria

refer 1.7.13 of MTC 03 Technical Description

- Portfolio - 10 marks

- Surprise project - 30 marks

- Main project - compliance to specification - 45 marks

- Main Project - build cost - 15 marks

NOTE: Marks will be entered into Worldskills CIS and converted to the 400-600 mark range.

3. Competition Requirements

- Each team will produce at the event:

  - A wind generator, pump and filter unit to meet the specification.

  - Section B of the portfolio including drawings and operating and maintenance instructions for the wind generator unit.

  - A surprise project - The surprise project will be released to competitors at 12:00 noon on the day immediately before the start of the competition. This project can consist of any combination of the skills outlined in the MTC technical description. This project will be selected by ballot by experts at expert meeting, just prior to competition.

### Activities to be done before the competition

  - Section A of the portfolio

  - Poster with team details

  - Materials may be rough cut into billets, or lengths but may not be machined to size. Burrs or sharp edges are not to be removed

  - Manufacture of electronic circuit boards, but these boards may not be mounted before the competition (if required)

  - Manufacture of the transportation enclosure to specification. This enclosure will be considered in the weight and assembly and also transportation enclosure tests.

NOTE: Purchased items are not to be modified in any way prior to the competition.

### Activities to be attempted during the competition

  - Section B of the portfolio, including drawings and manuals

  - Manufacturing of all components, including the frame of wind generator

  - All storage tanks

  - All shafts and mounting of drives

  - Electrical/electronic assembly – all individual components must be assembled during the competition

  - Manufacture guarding

  - Mechanical assembly of complete unit

  - Testing and commissioning the unit during the competition

  - Surprise project

- Equipment/Items provided by the organisers

  - All equipment, tooling and materials listed in updated infrastructure list – see MTC forum & MTC technical description

  - All materials and equipment deemed necessary for the surprise project.

  - Testing equipment for the wind generator unit and surprise project

- Items to be provided by the teams

  - Each team must provide and wear team identification (e.g. coloured shirts or armbands or country flag that indicate your country flag). These must be worn at all times during the competition.

  - It is the responsibility of the team to supply all the components and raw materials to manufacture the wind generator unit.

  - This would include the following items, but your design may require you to supply additional components or materials

    - Motors – motors must not be modified but may be modified during the competition

    - Batteries

    - electronic circuit board – un-mounted (circuit board components must be assembled on the board at the competition)

    - All electric and electronic components

    - Electric cables, connectors and couplings

    - Jigs, fixtures, formers and clamping devices

    - generator/pump/filter unit maintenance tooling.

    - All materials with which to construct the generator/pump/filter unit and all other associated equipment and consumables (LED’s, resistors, Circuit boards, sheet metal, screws, nuts, pipe flat bar etc)

    - Machining centre consumable tooling required for manufacturing wind generator components.

    - All hand tools, cutting tools and measuring equipment.

    - All personal protective equipment.

    - Other manufacturing equipment you require that is not listed in the infrastructure list

    - Lathe and mill tools and hand tools for manufacturing wind generator components

    - Transport enclosure

### Equipment and Materials not permitted

  - Laptop or portable computers

  - PDA’s eg Palm, IPAQ etc

  - Mobile phones

  - Memory sticks/MP3 players

  - Walkman radio/CD players

  - Electronic organisers/diaries

  - Wireless communication devices

  - Non approved CD’s or floppy discs – approval by chief expert or delegate is required for all CD’s and floppy discs

  - Any additional software not supplied by organisers unless approved by majority of experts

  - Purchased items are not to be modified in any way prior to the competition.

  - Teams will be controlled for components of the unit that they are not permitted to bring along. In other words, bringing purchased and self–made items that have to be made at the competition will be banned. A toolbox inspection will be conducted before the competition and any suspect components will be quarantined and banned from use during the competition.

  - During the competition duration, no tools, equipment, stationary, components, manuals, drawings or digital storage devices may be removed from or bought into the competition venue, unless approved by chief expert.

Note:

  - Digital or video cameras may be used during the competition to prepare documentation – memory must be erased by experts and camera will then be quarantined during the competition unless when required by the team. Camera must not to leave the competition site, during competition.

- Surprise Project – 30 marks

  - The surprise project main contain skills in manufacturing, design, assembly and quality not necessarily covered in the main project elements. The surprise project can consist of any combination of the skills outlined in the MTC technical description.

  - This project will be selected by ballot by experts just prior to competition and an evaluation scheme developed. Each expert will be required to submit a surprise project proposal.

  - The surprise project will be released to competitors at 12:00 noon on the day immediately before the start of the competition.

  - Teams can be awarded a maximum of 30 marks of marks in this section.

- Main project - compliance to specification - 45 marks

  - Wind Generator (maximum marks available = 10 marks)

# “The wind generator, must be portable, and be able to automatically change directions to suit the prevailing wind direction. All moving parts must be guarded to prevent any injury. The generator would charge a battery but only over a 12 hour period. The battery must be capable of illuminating a light and also to power the pump.”

# “The blades or turbine must be made during the competition but the electric generator or dynamo unit may be bought into the competition. All guards and supporting structure are to be made during the competition. The transport enclosure may be used as a mounting structure for the unit.”

# “The generator must have a visual indication of charging rate and also a visual indicator on the charged condition of the battery.”

    - Assessment Process – charge rate:

A fan is to be specified so that wind can be simulated. Fan should create wind in excess of 5m/sec (14km/hr) at a distance of 2 metres from fan. (Fan would be similar to a 250watt 600 diameter industrial pedestal fan.)

Each team will have a fan for trialling the wind generator and for assessment testing in Shizouka.

The wind generator to have test points or plugs for electrical test equipment, so that output voltage and current are able to be measured.

Fan (specification to be finalized) to be placed no closer than 2 metres from any part of the wind generator unit. Team may raise or lower the fan and ‘aim’ their fan to maximize wind effect. Output voltage and current to be tested and recorded.

# The generator would charge a battery but only over a 12 hour period. The battery must be capable of illuminating a light and also to power the pump.

      - 6 marks if equal to or over 100% capacity

      - 5 marks if equal to or over 90% capacity

      - 4 marks if equal to or over 80% capacity

      - 3 marks if equal to or over 70% capacity

      - 2 marks if equal to or over 60% capacity

      - 1 mark if equal to or over 50% capacity

      - 0 mark if less than 50% capacity

Evaluation procedure:

In optimum operating condition the output voltage and current of the wind generator unit will be tested. This test will be done 4 times at intervals of 5 minutes, and results averaged. The generator output will be calculated.

NOTE:

The generator must charge the battery over a 12 hour period, representing day hours.

The battery must have an output capable of operating the pump over 24 hours pumping 10L per hour, and also to operate the light continually over 12 hours representing night hours. (The generator will not be charging the battery during night hours)

Both battery and generator capacity will be assessed.

    - Assessment Process - automatically change direction:

Tests to be conducted at fan positions i.e. at 0o, 90o, 180o, 270o, 0o.  Output voltage and current to be tested and recorded at each test position.

Wind generator unit

0o

270o

180o

90o

# The wind generator, must be portable, and be able to automatically change directions to suit the prevailing wind direction.

      - 0.75 marks if operates at  90o

      - 0.75 marks if operates at 180o

      - 0.75 marks if operates at  270o

      - 0.75 marks if operates at  0o

Evaluation procedure:

Generator unit to be in normal operational position, fan then stopped and moved to new position. Fan to be started and wind generator must operate. Tests will be in the following order 0o, 90o, 180o, 270o and 0o. No team member may have contact with the wind generator for this test, but only able to have contact with the fan.

    - Assessment Process – charging indication:

- There is a visible charging indication

      - 0.5 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

There must be an indicator to indicate the battery is being charged.

    - Assessment Process – battery charge indication:

- There is a visible indication of battery condition

      - 0.5 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

There must be an indicator to indicate the charged level of the battery.

  - Pump (maximum marks available = 8 marks)

# “The pump must be capable of pumping 10 litres of fresh water per hour, and it will be considered that the water level is available at ground level.”

# “The pump may stop and start during pumping operation, and does not need to run constantly, but must be capable of pumping at least 10 litres per hour over 24 hours per day.”

    - Assessment Process – pump flow:

Pump to have external pickup pipe/tube. This pickup tube will be connected to a tray that has a maximum height of 50mm of water. Centre of outlet to pump to be 20mm from bottom of tray. (see details below) each team to supply own tray. Tray can be made before the competition and brought into the competition.

50mm

Outlet to pump

Overflow so that water level does not exceed a height of 50 mm

20mm

- The pump must be capable of pumping 10 litres of fresh water per hour.

      - 4 marks if equal to or 10 litres or more per hour

      - 3.2 marks if equal to or more than 9 litres per hour

      - 2.4 marks if equal to or more than 8 litres per hour

      - 1.6 marks if equal to or more than 7 litres per hour

      - 0.8 marks if equal to or more than 6 litres per hour

      - 0 marks if less than 6 litres per hour

Evaluation procedure:

20L storage tank to be empty then pump to run for 1 hour and the 20 litre catch tank is then emptied to check flow. Teams are permitted to continually fill the input tray for duration of test.

    - Assessment Process – pump control system – 2 hour test:

- The pump must be capable of pumping 10 litres of fresh water per hour. The pump may stop and start during pumping operation, and does not need to run constantly, but must be capable of pumping at least 10 litres per hour over 24 hours per day.

      - 2 marks if 10L per hour over 2 hour period

      - 0 mark if not 10L per hour

Evaluation procedure:

20L storage tank to be empty then pump to run for 2 hours and the 20 litre catch tank is emptied at end of each hour to check flow. Flow should be in excess of 10L per hour over this 2 hour test period. Teams are permitted to continually fill the input tray for duration of test.

    - Assessment Process – pump control system – 24 hour operation:

- The pump could operate for at least 24 hours pumping at least 10 L per hour

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

The voltage and current will be checked and pump power usage will be calculated. The wind generator charge rate and the battery capacity will be used to calculate length of time the pump can operate. Power requirements for the light must also be used in these calculations.

    - Assessment Process – storage tank:

- The unit has a 20L (litre) minimum storage tank.

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

The storage tank is to be made during the competition and the size will be calculated to check capacity.

The storage tank may have a larger capacity than 20 L.

Storage tank must have an outlet valve to drain the water from the tank.

NOTE:

The transport enclosure may be used for a mounting structure for the unit, but the storage tank must be made during the competition so if teams are using the transport enclosure as the storage tank, this storage tank must be made during the competition.

  - Light (maximum marks available = 3.5 marks)

# “The light must have a power of not less than 3 watts, and be capable of operating continuously for 12 hours from a single battery charge. The light must be able to operate at a distance of at least 5 metres from the generator unit.”

    - Assessment Process – light power:

- The light has a power output in excess of 3 watts and has a power on/off switch or button.

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

The light can be either a fluorescent, incandescent globe or LED type (or combination) but must be at least 3 watts. The voltage and current will be checked and power output of light will be calculated – power output of globes may be etched or printed onto globes. Light to be capable of ON/OFF operation

    - Assessment Process – light operating time:

- The light can operate continuously for at least 12 hours

      - 2 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

The voltage and current will be checked and power output of light will be calculated. (Power usage of the light of 3 watts per hour will be used in these calculations). The wind generator charge rate and the battery capacity will be used to calculate length of time the light can operate. Power requirements for the pump must also be used in these calculations.

10.3.c.  Assessment Process – 5 metre distance:

- The light can operate at least 5 metres from the wind generator unit

      - 0.5 mark if greater than 5 metres

      - 0 mark if less than 5 metres

Evaluation procedure:

Measure the distance the light can operate from the wind generator unit.

  - Filter (maximum marks available = 3 marks)

# “The filter system is to remove particles of dust and dirt and other contaminants from the water supply to a drinkable condition. The filter should be able to be easily cleaned or replaced, with an indicator to indicate when the filter flow is impeded by a buildup of contaminants.”

Note:

  - It is difficult to test the performance of the filter without specialized testing equipment.

  - The performance of the filter will not adversely affect outcome, but rather the assessment is whether there is a filter or not.

  - The performance of the filter elements could be adapted to suit requirements, i.e. coarse to remove large particulates like sand and silt, and down to fine/very fine to remove bacteria, which cause cholera and typhoid.

  - The filter element can be either disposable or able to be cleaned, but the time taken to clean or replace filter element will be assessed.

  - The filter will be ‘blocked’ or partially ‘blocked’ to test if the indicator is functioning.

    - Assessment Process – filter operation:

- A filter system is fitted and is operational.

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

A filter must be fitted and demonstrate that it would be capable of removing contaminates from water. The size of the filter element will not be considered but filter element must be capable of filtering.

    - Assessment Process – filter element cleaning/replacement:

- The filter system or element is easily cleaned or replaced.

      - 1 mark if able to clean and/or replace filter system or element within 1 minute

      - 0 mark if time taken is greater than 1 minute.

Evaluation procedure:

The wind generator unit is to be placed in operational condition, at a distance of 1 metre away from team. The filter system or the filter element is to be removed from the wind generator unit and placed at a distance at least 1 metre from the wind generator unit. The filter element is then assembled using only the maintenance tools supplied from the transportation enclosure. A replacement filter system may be used for re-assembly.  The filter must then demonstrate operation.

The assembly time will be timed.

All three (3) members of the team can be involved in the replacement of the filter system or element.

    - Assessment Process – filter indicator:

- An indicator or meter indicates if water flow through the filter is impeded or blocked. The meter must indicate if filter is clean or if filter requires cleaning

      - 1 mark if  indicator is fitted and operational

      - 0 mark if no indicator is fitted or does not indicate correctly when flow is impeded

Evaluation procedure:

Flow through the filter system will be ‘blocked’ or ‘partially blocked’ and indicator must indicate there is a restriction to flow. There must be indicator positions for ‘clean’ and ‘requires cleaning’ conditions of the filter (Other words can be used to indicate the filter condition).

  - Weight (maximum marks available = 2 marks)

“The unit must be lightweight”

    - Assessment Process – weight:

Each unit packed in transportation enclosure, complete with maintenance tools will be weighed.

      - 2 marks to lightest weight for complete unit, then reduce 1 mark per 5kg (proportional) heavier than lightest weight

Evaluation procedure:

The wind generator unit complete with guards and transportation enclosure, maintenance tools, battery charger, repair and maintenance manual and operation manual will be weighed. If all components are not presented for weighing no marks will be awarded for this section.

  - Assembly (maximum marks available = 4 marks)

“The unit must be lightweight and easy to transport and fit within a transport enclosure of combined length, width and height of not greater than 1800mm (all maximum sizes of enclosure to be added).

“The unit should be quick to remove from the transport enclosure and set up ready for operation, and also to pack up and to replace the unit into the transport enclosure. The enclosure may be used as a mounting structure or container for the unit.”

“The battery must be disconnected when packing the unit into the transport container, and it will be considered that the battery will be transported in an upright position.”

    - Assessment Process – assembly time:

- Assembly time for wind generator unit

      - 3 marks to fastest time then reduce 1 mark (proportional) per minute slower than fastest time

Evaluation procedure:

The wind generator unit is to be placed in the transportation enclosure with all maintenance tools and manuals a distance of 1 metre away from team. The enclosure must be closed and latched or secured. The wind generator unit is to be removed from the enclosure and assembled into its normal operating configuration using only the maintenance tools supplied from the transportation enclosure. The wind generator unit must demonstrate operation.

The assembly time will be timed.

All three (3) members of the team can be involved in the assembly of the wind generator unit.

    - Assessment Process – size:

- The wind generator unit fits in the specified envelope.

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation procedure:

The wind generator unit is to be placed in the transportation enclosure of combined length, width and height of not greater than 1800mm (all maximum sizes of enclosure to be added, i.e. add length, width and height together) with all maintenance tools and manuals.

All three (3) members of the team can be involved in the placing the wind generator unit into the enclosure.

  - Transportation Enclosure (maximum marks available = 1 mark)

“Transport Enclosure: The transport enclosure must be capable of withstanding a static load of 200N acting over an area of 75 mm diameter. This transport enclosure can be made before the competition, but will be included in the testing.”

Note: Manufacture of the transportation enclosure can be done prior to the competition.

The enclosure test will be carried out as first performance test, and if the wind generator unit is damaged no maintenance will be allowed. This may have an effect on further testing of your wind generator unit.

    - Assessment Process – applied force:

- Applied force – if enclosure is damaged during testing and affects the unit inside

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation process: This test will be carried out as the first performance test. The enclosure will be tested by applying a static force of 200N from a 75mm diameter test bar at 3 locations on the enclosure. It may deflect, but should not damage the wind generator unit or any equipment within the enclosure. Note if your wind generator unit is damaged during this test it could have an effect on further testing of the wind generator unit.

  - Guarding (maximum marks available = 1 mark)

“All guards and supporting structure are to be made during the competition”

    - Assessment Process – exposed electrical connections:

- Exposed Electrical connections

      - 1 mark if no exposed connections

      - 0 mark if exposed electrical connection

Evaluation process: Check if any exposed electrical connection has any exposed wire not fitted with insulation or a suitable covering

  - Maintenance and Repair (maximum marks available = 2.5 marks)

“The Wind generator unit should be easily maintained and/or repaired with quick-fit or plug-in units with minimum skill and tooling requirements. Any tooling for repair and/or maintenance should be contained and fixed within the enclosure.”

    - Assessment Process – maintenance time:

- 2 marks to fastest team, then reduce 0.5 mark per minute (proportional) slower than fastest time

Evaluation process:- The following tests will be performed individually and the total time calculated, only tools specified in the maintenance manual and contained in the enclosure or attached to the wind generator unit will be allowed.

TEST 1: The battery/batteries is to be completely removed from the wind generator unit in its normal operating configuration, placed statically 1 metre away and then replaced and connected. If multiple batteries are used all the batteries are to be removed but may remain connected to each other. The wind generator unit must then demonstrate correct operation.

TEST 2: The pump/motor unit to be completely removed from the wind generator unit, placed statically 1 metre away, and then replaced and connected. The pump unit must then demonstrate correct operation.

    - Assessment Process – maintenance tools:

- If tools are not fixed within the enclosure

      - 0.5 mark if acceptable

      - 0 mark if not acceptable or is incomplete

Evaluation process: All tools specified in the maintenance manual must be fixed securely within the enclosure and only these tools are used during maintenance and repair test

  - Drawings (maximum marks available = 7 marks)

“Compile a dossier of the complete process.”

Drawings

From MTC technical Description

1.4.1 	Design - Ability to interpret, construct, and modify engineering CAD drawings, to work with 3D modelling, and to convert both to CAM. Use CAM to generate toolpaths using a suitable postprocessor.

Drawing standard will be ISO-A or ISO-E depending on country involved.

NOTE: All drawings to be created in electronic format and paper copies included in Section B of the portfolio

    - Assessment Process – 3D assembly drawing:

- 3D Assembly drawing

      - 2 marks if acceptable

      - 0 mark if not acceptable or is incomplete

Evaluation process: The drawing should be at least 90 % complete and correct and:

    - have title block with:

    - Name

    - Scale

    - Drawing number

    - Projection system

    - border

    - have parts/material list

    - parts/material list to be referenced to drawing

    - Assessment Process – 2D detail drawings:

- 2D detail drawings

      - 4 marks if acceptable

      - 0 marks if not acceptable or is incomplete

Evaluation process: The drawings should be at least 90 % complete and correct and:

    - have title block with:

    - Name

    - Scale

    - Drawing number

    - Projection system

    - material

    - drawing border

    - have appropriate views

    - dimensioned and toleranced with surface finish symbols where appropriate

    - sections (if required)

    - notes (if required)

    - guards are not required to be drawn

    - Assessment Process – electrical/electronic:

- Electronic/electrical wiring circuit

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation process: The drawing should be at least 90 % complete and correct and:

    - have title block with:

    - Name

    - Drawing number

    - drawing border

    - standard electrical/electronic symbols with component values - block representation also accepted if electrical/electronic graphics library not available

Judges Note:

When assessing the drawings using the above criteria, it is sufficient to award marks for the information and interpretation when it comes up to the minimum requirements, rather than consider the actual detail contained within it.

  - Manuals (maximum marks available = 2 marks)

From MTC technical Description

1.3.8 	Documentation - Document the process including header page, index, and descriptive overview of the task, hard copy of any programs, instructions for setting up and assembly, and any relevant drawings.

This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos. A paper copy of this manual is to be printed and included in Section B of the portfolio document.

    - Assessment Process – operation manual:

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation process: The operation manual will be evaluated by a team of three (3) experts and should consist of:

- Index/table of contents with page numbers

- List of items supplied with the wind generator unit, including the transportation enclosure, maintenance tools and manuals

- Function and operation of all controls/switches/indicators including battery condition, power isolation, lights etc.

- General specification and performance of wind generator unit, including weight, maximum and minimum design wind speed, weight, power output, etc.

- Safety considerations when using the wind generator unit

- General hints on using wind generator unit

    - Assessment Process –  Maintenance & repair manual

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation process: The maintenance and repair manual will be evaluated by a team of three (3) experts and should consist of:

- Index/table of contents with page numbers

- List of maintenance tools supplied

- Instructions to include

- assembly and packing wind generator unit into transportation enclosure

- replacement of battery/batteries

- replacement of pump and motor unit(integrated unit)

- replacement of light/globe

- replacement or cleaning of filter

- Lubrication of wind generator unit (if required)

  - Manufacturing Schedule (maximum marks available = 1 mark)

The original manufacturing plan will need to be modified after the surprise project is known to each team and must reflect the manufacturing process undertaken by each team.

This manufacturing plan is to be done in electronic format with paper copy included in Section B of portfolio document

    - Assessment Process – manufacturing schedule:

- manufacturing schedule to include manufacturing processes of the surprise project

      - 1 mark if acceptable

      - 0 mark if not acceptable

Evaluation process: The manufacturing schedule manual will be evaluated by a team of three (3) experts and must have the manufacture of the surprise project included.

- Material & Production costs – 15 marks

NOTE: all costs to be calculated in Japanese Yen (JPY)

  - Working hours: 					¥3,000/person/hour

NOTE: When one member of the team is working, all team members will be costed therefore team cost would be ¥9,000/hour

Labour and equipment costs when working on the surprise project are to included with main project labour and equipment costs.

  - Additional price for equipment use:

For using a conventional mill and lathe:				¥2,300/hour

For using the CNC mill  						¥3,800/hour

For using workshop equipment, e.g. welding, computer, grinding, sheet metalwork, drilling and saw							¥1,500/hour

Consultant or training							¥6,000/hour

The minimum booking time for each machine or consultancy is fifteen (15) minutes.

NOTE:

After using a machine, the machine must be cleaned, i.e. swarf removed from working area of the machine. Each machine will be inspected by an expert after each team’s use of that machine and a penalty of ¥750 (representing 15 minutes of cleaning time) will be applied if machine is not cleaned. If a machine is considered not cleaned a group of three (3) experts will be called to inspect that machine. Their decision is final.

Equipment use cost only to be applied to equipment supplier by Japanese organisers. No cost for using portable equipment provided by teams.

  - Raw materials:

Each team will provide weight of raw materials used and also cost of raw material using cost per kilogram listed below.

The list of raw material details including weight and cost is to be presented in Spreadsheet format and to be included in Section A of the Portfolio

Cost for Raw Materials to be applied:

- Mild steel – ¥70/kg

- Aluminium - ¥106/kg

- Brass - ¥375/kg

- Stainless steel - ¥285/kg

- Plastic - ¥200/kg

- For each purchased item without a catalogue price or internet e-catalogue printout and receipt, an expert will search for a typical catalogue price and that will be charged at double the price.

- Materials bought into the competition may be rough cut or sawn but not machined to finished size – a cutting cost will be incurred if material is in finished size.

- Sheet metal may be cut to a finished size and may be folded or bent and bought into the competition. Each straight cut will incur a cost of ¥750 for each cut and a fold or bend will incur a cost of ¥750 for each bend. All sheet materials will be inspected before the competition.

- Pipe or tube, flats or rounds may be bent and bought into the competition but a cost of ¥750 for each bend will be applied.

- Materials that are bought into the competition and are not cut to size, bent or folded will not incur the costs above but will be costed at workshop rates.

  - Purchased items:

For each used purchased item a receipt or a current catalogue price (without discounts or goods and services taxes applied). A printout of a website or e-catalogue is acceptable if website address and date are included on printout.

The list of purchased items and their costs is to be presented in Spreadsheet format and to be included in Section A of the Portfolio

  - The motor and generator basic units must not be modified in any way before the competition, and their cost will not be considered in project cost. Both the motor and the generator can be modified during the competition.

- Geared motors are permitted but must not have any pulley, sprocket or drive coupling pre assembled on the shaft.

- All electronic/electric control components, eg motor controllers. Individual components may be bought into the competition and must not be assembled. All circuit boards must be assembled during the competition.

NOTE:

If any raw material or purchased item is used in the main project and not included in either of the lists above, or costs not included in the lists, the cost will be estimated by a team of experts and double the cost will be added.

  - Cost Calculations.

Step 1

The total cost of equipment, materials and labour will be modified by project compliance to specification.

Final Build cost = 	            total cost	_______

% compliance to specification

Note: see Section 10 for ‘Compliance to Specification’

Example:

- If total cost is ¥200,000 and compliance is 100% then build cost would be ¥200,000

- If total cost is ¥200,000 and compliance is 80% then build cost would be ¥250,000

- If total cost is ¥200,000 and compliance is 60% then build cost would be ¥333,333

- If total cost is ¥200,000 and compliance is 40% then build cost would be ¥500,000

- If total cost is ¥200,000 and compliance is 20% then build cost would be ¥1,000,000

- If total cost is ¥200,000 and compliance is 0% then no marks awarded for cost section.

Step 2

The build costs will then be compared between teams. The cheapest team will receive 15 marks. For every 10% more expensive than the cheapest cost, deduct 1 mark proportionally.

- Portfolio Assessment – 10 marks

The portfolio will consist of two sections, Section A and Section B

Section A: This section of the portfolio will be presented prior to the competition and will be assessed during the competition.

NOTE: This section is to be produced before the competition.

Section B: This section of the portfolio includes documentation prepared during the competition.

NOTE: This section is to be produced during the competition. The time taken to complete Section B will be costed as a part of the main project.

  - Section A:

This section of the portfolio is to be presented prior to the competition start. This portfolio to be in electronic format but presented in both hard copy and electronic form and to include:

- team member details

- team preparation

- 3D model drawing of the vehicle

- poster display

- freehand sketches or drawings

- calculations

- computer generated 3 day competition manufacturing schedule

- lists of all materials and components

    - Details of team and team members

- your country

- name,

- date of birth,

- home address,

- educational & vocational background

- employer’ name and business address

    - Team preparation

- Worldskills regional and national competitions and results

- your specific role or skills you provide to the team

- details of any training undertaken by individuals or the team in preparation for the event.

    - 3D model drawing of the complete wind generator unit

A 3D drawing of the complete unit in electronic drawing/modelling printed format

- The 3D model should contain all mechanical parts that are present in the unit itself. The model parts should be geometrically identical to the machined parts. For all the purchased items, only the rough outside dimensions are decisive.

NOTE: the electronic form of this model will not be permitted to be bought into the competition.

    - A poster display 1000mm x 600mm (minimum size) detailing each team member’s details and their employers, and a description of the project solution, including a 3D model of their vehicle, which will be presented prior to the competition and displayed to the public during the competition.

    - Freehand sketches

- Freehand sketches of any mechanical components for the project, but these must not be in an electronic format, but rather in a freehand form. Copies of these sketches may be used during the competition and also to create the electronic version of the assembly and detail drawings during the competition.

- Freehand sketches of the electrical circuit. These sketches may be used to create the electronic version of the electrical circuit drawings during the competition.

    - Design calculations

Design calculations including:

- Generator/dynamo size

- Battery Size - size

- Weight of the complete unit, excluding enclosure

- Weight of the enclosure

- Proposed cost of completed vehicle

NOTE: these calculations may be written in the language of the team’s country

    - Computer generated 3 day competition manufacturing schedule

- A manufacturing schedule for the manufacture of the project, also including the activities involved with Section B of the portfolio, is to be presented in a printout format.

This manufacturing plan will need alteration after the surprise project is known to the teams.

NOTE: this schedule may be written in the language of the team’s country

    - Lists of raw materials.

Each team will provide weight of raw materials used and also cost of raw material using cost per kilogram listed below.

The list of raw material details including weight and cost is to be presented in Spreadsheet format and to be included in Section A of the Portfolio

Cost for Raw materials to be applied:

- Mild steel – ¥70/kg

- Aluminium - ¥106/kg

- Brass - ¥375/kg

- Stainless steel - ¥285/kg

- Plastic - ¥200/kg

    - Purchased items:

For each used purchased item a receipt or a current catalogue price (without discounts or goods and services taxes applied). A printout of a website is acceptable if website address and date are included on printout.

A list of purchased items and their costs is to be presented in Spreadsheet format and to be included in Section A of the Portfolio

NOTE: this schedule may be written in the language of the team’s country

  - Section B.

This section of the portfolio is to be completed during the competition and presented as an indexed word processed document. This section should be presented in one folder or binder and include:

- Assembly drawing of the vehicle

- 2D detailed drawings of all manufactured components

- Electrical/electronic circuit diagrams

- Updated manufacturing plan

- Operating instructional manual

- Maintenance & repair manual

    - Assembly drawing of complete generator /pump/ filter unit

This drawing should include a list of all the components with a parts list with reference to detail drawings

- This drawing is to be done in electronic format with paper copy printout included in portfolio document

    - 2D detailed drawings of all manufactured components

All manufactured components to be drawn and referenced to the assembly drawing.

- These drawings are to be done in electronic format with paper copies included in portfolio document

    - Electrical/electronic circuit diagrams

Complete circuit diagrams detailing wiring details and circuit board layout (if used in your project)

- This drawing is to be done in electronic format with paper copy included in portfolio document

    - Updated manufacturing schedule

The original manufacturing schedule will need to be modified after the surprise project is known to each team and must reflect the manufacturing process undertaken by each team.

- This manufacturing plan is to be done in electronic format with paper copy included in portfolio document

NOTE: a printed copy of the original manufacturing schedule as presented in Section A of the portfolio will be available.

    - Operating instructional manual.

This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos.

A paper copy of this manual is to be printed and included in the portfolio document.

NOTE: this manual may be written in the language of the team’s country

    - Maintenance & repair manual.

This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos.

A paper copy of this manual is to be printed and included in the portfolio document.

NOTE: this manual may be written in the language of the team’s country

    - Section A – Marks Summary

| Section A | Marks | Acceptable | Not Acceptable |
| --- | --- | --- | --- |
| Team member details | 0 or .25 |  |  |
| Team preparation | 0 or .25 |  |  |
| 3D model drawing of the wind generator unit including pump and filter system | 0 or 2 |  |  |
| Poster display | 0 or .5 |  |  |
| Freehand sketches of components of the wind generator unit | 0 or 1 |  |  |
| Calculations | 0 or 1 |  |  |
| Original 4 day manufacturing schedule – not including surprise project | 0 or .25 |  |  |
| Lists of all materials and components used and receipts or catalogue costs for all materials and components used – to be in spreadsheet format | 0 or 1.5 |  |  |
| Presented in folder or as bound document | 0 or .25 |  |  |
| Possible marks | 0 to 7 |  |  |

    - Section B – Marks Summary

| Section B | Marks | Acceptable | Not Acceptable |
| --- | --- | --- | --- |
| Assembly drawing | 0 or .5 |  |  |
| 2D detailed drawings | 0 or .5 |  |  |
| Electrical/electronic wiring diagrams | 0 or .5 |  |  |
| Updated manufacturing schedule | 0 or .25 |  |  |
| Operation manual | 0 or .5 |  |  |
| Maintenance & repair manual | 0 or .5 |  |  |
| Printed and presented in folder | 0 to .25 |  |  |
| Possible marks | 0 to 3 |  |  |

| Total possible marks for portfolio (your mark out of possible 10 marks) |  |
| --- | --- |

Judges Note:

When assessing the portfolio using the above criteria, it is sufficient to award marks for inclusion of the information when it comes up to the minimum requirements, rather than consider the actual detail contained within it.

Marks are awarded for each item as if acceptable – full marks or if not acceptable - zero marks (there will be no graduated marks)

The minimum requirements will be decided by the experts prior to the competition and fully documented before any assessment of the portfolio.