Manufacturing Team Challenge

Helsinki 2005

# Competition Information Pack

Contents								Page

1. General Competition Information	2
2. Project Brief	4
3. Outline of Project Requirements	5

4.  Portfolio requirements	5

5.  Items provided by the organisers 	10

6. Items to be provided by the teams.	10

### 7. Activities to be carried out before the competition	11

### 8. Activities to be carried out during the competition	11

### 9. Marking Scheme – overview	12

### 10. Portfolio marking scheme	15

### 11. Marking scheme for design and production costs	17

### 12. Marking scheme for practical purposes	18

### a) performance evaluation	18

### b) battery and charge systems	20

### c) weight	22

### d) assembly	22

### e) transportation enclosure	23

### f) vehicle safety requirements	23

### g) maintenance and repair	24

### h) documentation	25

### i) manuals	25

### j) manufacturing schedule	26

### 1. General Competition Information

The current MTC technical description should be available to all experts, competitors and team mentors. The technical description is available for download from the WorldSkills web site (www.worldskills.com)

Some of the quotes from the current technical description include (numbers refer to technical decription):

1.1.4 	Every expert and competitor must know this Technical Description.

1.1.5 	In the event of any conflict within the Technical Descriptions, the English version will take precedence.

1.1.3 	When one member of the team is working all team members will be costed. Teams need a team identification (e.g. coloured shirts) and armbands that indicate your country (e.g. country flag)

1.2.1 	The competitors/teams must be able to:

- Effectively manage a team for optimum performance.

- Analyse and control production costs within the set guidelines.

- Comply with occupational health and safety procedures.

- Organise skill improvement through training sessions.

- Design any jigs fixtures and processes to complete the allotted task(s).

- Generate any automation programs needed to complete the allotted task(s).

- Machine the designated part(s), jigs, and/or fixtures using conventional or CNC methods to the required tolerances.

- Carry out assembly of any designated components using automated methods.

- Utilise modern problem solving techniques in achieving the desired result.

- Optimise the complete process in order that cycle times are minimised.

- Compile a dossier of the complete process.

- Join processes.

1.2.2 	There will be three (3) competitors per MTC team.

1.3.1 	The task(s) should be designed in the year preceding the Competition so that the team composition, selection, and training may proceed and the major equipment list compiled.

1.2.3 	Competitors are not to exceed 25 years of age in the year of the competition.

1.2.4 	The total working time for the project will be between 18 and 22 hours.

The competition will take place over four (4) days in Helsinki, Finland from Thursday 26th May to Sunday 29th May 2005

1.7.5 	Marking will be objective in accordance with the marking scale.

• Only Form 6 will be required for submission of marks.

• Marking is to be entered after each section has been completed, and a program has been developed for computer calculation after time and task data has been entered.

1.7.6 	All marking will be done on a spreadsheet with pre-programmed formulae.

1.7.7 	Marking will be on the basis of product cost and will include such things as competitors’ time, materials used, parts used, any consulting fees, training required, and machine tool and tooling costs. Required tolerances must be met for a result to be valid.

1.7.8 	There may be other forms of assessment for sub-categories of the main project such as cycle time where applicable which may also translate to cost per product item made and inclusion of specific items of documentation

1.7.9 	A spreadsheet program has been written for computer calculation of results upon entry of the raw data such as hours, material item costs, etc and which can be easily modified to suit any project.

1.7.10 	Multiple awards may be made.

1.7.11 	The portfolio has to be carried out during the competition and will be costed.

2. Project Brief

Personal Transportation Vehicle

The vehicle is intended for use in the inner city as a means of reducing waste, pollution and heavy congestion caused by motor vehicles. It must be designed and manufactured to meet the needs of the environment in which it is proposed to operate.

The vehicle will operate by battery and must be designed to re-charge within a typical working day of 8 Hours. However, it should also be capable of fast charging within 2 hours. Evidence of initial battery conditions and charging indication must be provided. The vehicle will have both speed and braking controls and be designed to achieve maximum speed of 6Km/h, climb an incline of 11,3 degrees (1 in 5) ten times in an hour with same battery charge and has to do controlled declines. It must be capable of transporting one person up to 75Kg and have capacity to carry 5Kg of luggage attached to the vehicle, within a maximum load space of 20cm x 20cm x 15cm.  The overall turning circle must be within a 2,0 metre diameter.

The Vehicle must incorporate systems to cover night-time use, these must be visible from both the rear and front of the vehicle and provide visible illumination from a distance of 50m during dusk and darkness hours. It will also include a method of isolating the power when not in operation. An audible and visual warning will be required when the battery power runs low. The operational position should be adjustable to cover a range of differing height positions. The vehicle should be readily assembled within a target time of 2 minutes and transported within an enclosure no bigger than 1m x 0.3m x 0.3m. The complete unit should be light and portable. The enclosure of the unit has to be able to withstand handling forces of 500 N. The materials and components used throughout should be suitable for the purpose and the environment in which they will be installed.

The Vehicle should be easily maintained and/or repaired with quick-fit or plug-in units with minimum skill and tooling requirements. Any tooling for repair and/or maintenance should be contained and fixed within the enclosure.

NOTE: This project was selected by a majority ballot closing on 2nd July 2004 with results announced on the MTC Forum on 5th July

3. Outline of Project Requirements

Each team will produce at the event:

- A vehicle to meet the specification.

- A portfolio showing prior research and team preparation and also the manufacturing processes and operating instructions for the vehicle.

- A surprise project - this will be explained at the event on the first day. This project can consist of any combination of the skills outlined in the MTC technical description. This project will be selected by ballot by experts just prior to competition.

4.  Portfolio requirements

The portfolio will consist of two sections, Section A and Section B

Section A is to be completed prior to the competition and the Section B is to be completed during the competition.

The time taken to complete Section B will be costed as a part of the project.

Section A: This section of the portfolio is to be presented prior to the competition start. This portfolio to be in electronic format but presented in both hard copy and electronic form and to include:

- team member details

- team preparation

- 3D model drawing of the vehicle

- freehand design calculations and sketches

- computer generated 4 day competition manufacturing schedule

- lists of materials and components used with catalogue prices

- evidence of material and component costs

A poster display 1000mm x 600mm (minimum size) detailing each team member’s details and their employers, and a description of the project solution, including a 3D model of their vehicle, which will be presented prior to the competition and displayed to the public during the competition.

a) Details of team and team members

- your country

- name,

- date of birth,

- home address,

- educational & vocational background

- employer’ name and business address

b) Team preparation

- your specific role or skills you provide to the team

- details of any training undertaken by individuals or the team in preparation for the event.

c) 3D model drawing of the vehicle

A 3D model of the complete unit in electronic drawing/modelling format

- The 3D model should contain all mechanical parts that are present in the unit itself. The model parts should be geometrically identical to the machined parts. For all the purchased items, only the rough outside dimensions are decisive.

NOTE: the electronic form of this model will not be permitted to be bought into the competition.

d) Freehand design calculations and sketches

- Freehand sketches of any mechanical components for the project, but these must not be in an electronic format, but rather in a freehand form. Copies of these sketches may be used during the competition and also to create the electronic version of the assembly and detail drawings during the competition.

- Freehand sketches of the electrical circuit. These sketches may be used to create the electronic version of the electrical circuit drawings during the competition.

Design calculations including:

- Power requirements for the vehicle

- Motor Size - torque and speed

- Drive train – drive ratio and size

- Battery Size - size

- Predicted operating range

- Strength calculation of the transportation enclosure

- Weight of the vehicle, excluding enclosure

- Weight of the enclosure

- Proposed cost of completed vehicle

NOTE: these calculations may be written in the language of the team’s country

e) Computer generated 4 day competition manufacturing schedule

- A manufacturing schedule for the manufacture of the project, also including the activities involved with Section B of the portfolio, is to be presented in a printout format.

Note: that this plan will need alteration after the surprise project is known to the teams.

NOTE: this schedule may be written in the language of the team’s country

f) Lists of materials and components used, with catalogue prices as evidence of material and component costs

- A list of all purchased items including all required materials, fasteners, drive components, lubricants, and electronic/electrical components, with their original receipts plus original copy of up-to-date catalogue with price lists OR an up-to-date internet e-catalogue printout with price list must be presented for all components. The internet e-catalogue must have the internet address printed on this printout. Discounts are not permitted.

NOTE:

  - This list to be presented in a spreadsheet table format similar to that created by Microsoft Excel.

  - This list will be used to calculate the cost of components and raw material used in the project. All items in the project are to be listed.

  - These lists may be written in the language of the team’s country

Section B. This section of the portfolio is to be completed during the competition and presented as an indexed word processed document. This section should be presented in one folder or binder and include:

- Assembly drawing of the vehicle

- 2D detailed drawings of all manufactured components

- Electrical/electronic circuit diagrams

- Updated manufacturing plan

- Operating instructional manual

- Maintenance & repair manual

a) Assembly drawing of vehicle

This drawing should include a list of all the components with a parts list with reference to detail drawings

- This drawing is to be done in electronic format with paper copy included in portfolio document

b) 2D detailed drawings of all manufactured components

All manufactured components to be drawn and referenced to the assembly drawing.

- These drawings are to be done in electronic format with paper copies included in portfolio document

c) Electrical/electronic circuit diagrams

Complete circuit diagrams detailing wiring details and circuit board layout (if used in your project)

- This drawing is to be done in electronic format with paper copy included in portfolio document

d) Updated manufacturing schedule

The original manufacturing schedule will need to be modified after the surprise project is known to each team and must reflect the manufacturing process undertaken by each team.

- This manufacturing plan is to be done in electronic format with paper copy included in portfolio document

NOTE: a printed copy of the original manufacturing schedule as presented in Section A of the portfolio will be available.

e) Operating instructional manual. This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos. A paper copy of this manual is to be printed and included in the portfolio document.

This manual should include:

- Vehicle specifications

- Unpacking and packing the vehicle into the transportation enclosure

- All operating controls

- Safety precautions

- Electrical isolation switches

- Packing and unpacking the vehicle into the transport enclosure.

f) Maintenance & repair manual. This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos. A paper copy of this manual is to be printed and included in the portfolio document.

This manual should include:

- Replacement of battery

- Replacement of all wheels

- Replacement of motor

- Troubleshooting guide

- Recharging procedure

5.  Items provided by the organisers

- All materials and equipment deemed necessary for the surprise project.

- Testing equipment for the vehicle and surprise project

6. Items to be provided by the teams

Each team must provide and wear a team identification (e.g. coloured shirts) and armbands that indicate your country (e.g. country flag). These must be worn at all times during the competition.

It is the Responsibility of the team to supply all the components and raw materials to manufacture the vehicle.

This would include the following items, but your design may require you to supply additional components or materials

- Motors

- Batteries

- Wheels and Tyres

- Brakes and cables

- Seat (standard item unmodified)

- Bearing or bushes (unmodified)

- Sprockets, gears, couplings and belts (as supplied and must not be altered)

- electronic circuit board – unmounted (circuit board components must be assembled on the board at the competition)

- All electric and electronic components (NOTE: motor controllers and battery chargers must be assembled during the competition)

- Electric cables, connectors and couplings

- Jigs, fixtures, formers and clamping devices

- Vehicle maintenance tooling.

- All materials with which to construct the vehicle and all other associated equipment and consumables (LED’s, resistors, Circuit boards, sheet metal, screws, nuts, pipe flat bar etc)

- Machining centre consumable tooling required for manufacturing vehicle components.

- Battery charger

- Tyre pump – if required

- All hand tools, cutting tools and measuring equipment.

- All personal protective equipment.

- Other manufacturing equipment you require that is not listed in the infrastructure list

- Lathe tools and hand tools for manufacturing vehicle components

- Transport enclosure

### 7. Activities to be carried out before the competition

- Section A of the portfolio

- Poster with team details

- Materials may be rough cut into billets, or lengths but may not be machined to size. Burrs or sharp edges are not to be removed

- Manufacture of electronic circuit boards, but these boards may not be mounted before the competition (if required)

- Manufacture of the transportation enclosure to specification (maximum outside dimensions of enclosure to be 1.0m x 0.3m x 0.3m). This enclosure will be considered in the weight and assembly and also transportation enclosure tests.

NOTE: Purchased items are not to be modified in any way prior to the competition.

### 8. Activities to be carried out during the competition

- Section B of the portfolio, including drawings and manuals

- Manufacturing of all components, including the chassis or frame of vehicle

- All shafts and mounting of drives

- Electrical/electronic assembly - motor controllers and battery chargers must be assembled during the competition

- Manufacture vehicle guarding

- Mechanical assembly of complete unit

- Testing the unit

- Surprise project

# 9. Marking Scheme - Overview

The evaluation of this competition will be in three (3) sections with the following possible marks.

Portfolio 				10%

Vehicle 				60%

Surprise project			30%

Portfolio

Teams satisfying all requirements of the portfolio evaluation will gain 10%

Vehicle

The team with the cheapest build cost for the vehicle after cost penalties for non compliance is included will gain 60% for this section.

Teams with a higher build cost will receive fewer marks when compared to the cheapest build cost.

NOTE

  - For each purchased item without a catalogue price or internet e-catalogue printout and receipt, an expert will search for a typical catalogue price and that will be charged at double the price.

- Materials bought into the competition may be rough cut or sawn but not machined to finished size – a cutting cost will be incurred if material is in finished size.

- Sheet metal may be cut to a finished size and may be folded or bent and bought into the competition. Each straight cut will incur a cost of €5 for each cut and a fold or bend will incur a cost of €5 for each bend. All sheet materials will be inspected before the competition.

- Pipe or tube, flats or rounds may be bent and bought into the competition but a cost of €5 for each bend will be applied.

- Materials that are bought into the competition and are not cut to size, bent or folded will not incur the costs above but will be costed at workshop rates.

- Mechanical components for the speed controls and braking system may be bought into the competition, but must not be assembled, ie brake lever/pedal and brake calipers must not be assembled with cables or rods.

- Geared motors are permitted but must not have any pulley, sprocket or drive coupling pre assembled on the shaft.

- Integral wheel and drive sprocket assembly is not permitted to be bought into the competition.

- All electronic/electric control components, eg speed controls, braking, charging, lighting and charge indication must not be pre-made or preassembled. Individual components may be bought into the competition and must not be assembled. All circuit boards must be assembled during the competition.

- After using a machine, the machine must be cleaned, ie swarf removed from working area of the machine. Each machine will be inspected by an expert after each team’s use of that machine and a penalty of €5 (representing 15 minutes of cleaning time) will be applied if machine is not cleaned. If a machine is considered not cleaned a group of three (3) experts will be called to inspect that machine. Their decision is final.

### Equipment and Materials not permitted

- Laptop or portable computers

- PDA’s eg Palm, IPAQ etc

- Mobile phones

- Digital or film cameras

- Memory sticks/MP3 players

- Walkman radio/CD players

- Electronic organisers/diaries

- Wireless communication devices

- Non approved CD’s or floppy discs – approval by chief expert or delegate is required for all CD’s and floppy discs

- Any additional software not supplied by organisers unless approved by majority of experts

- Purchased items are not to be modified in any way prior to the competition.

- Teams will be controlled for components of the unit that they are not permitted to bring along. In other words, bringing purchased and self–made items that have to be made at the competition will be banned. A toolbox inspection will be conducted before the competition and any suspect components will be quarantined and banned from use during the competition.

- During the competition duration, no tools, equipment, stationary, components, manuals, drawings or digital storage devices may be removed from the competition venue, unless approved by chief expert.

Surprise Project

The surprise project main contain skills in manufacturing, design, assembly and quality not necessarily covered in the main project elements. The surprise project can consist of any combination of the skills outlined in the MTC technical description.

This project will be selected by ballot by experts just prior to competition and an evaluation scheme developed. Each expert will be invited to submit a surprise project proposal.

Teams can be awarded a maximum of 30% of marks in this section.

10. Portfolio Marking Scheme

Section A: This section of the portfolio will be presented prior to the competition and will be assessed during the competition

| Section A | Marks | Acceptable | Not Acceptable |
| --- | --- | --- | --- |
| Team member details | 0 or 1 |  |  |
| Team preparation | 0 or 1 |  |  |
| 3D model drawing of the vehicle | 0 or 1 |  |  |
| Poster display | 0 or 1 |  |  |
| Freehand sketches of components of the vehicle | 0 or 1 |  |  |
| Calculations | 0 or 1 |  |  |
| Original 4 day manufacturing schedule | 0 or 1 |  |  |
| Lists of all materials and components used | 0 or 1 |  |  |
| Receipts for all materials and components | 0 or 1 |  |  |
| Presented in folder | 0 or 1 |  |  |
| Possible marks | 0 to 10 |  |  |

Section B: This section of the portfolio will be completed during the competition

| Section B | Marks | Acceptable | Not Acceptable |
| --- | --- | --- | --- |
| Assembly drawing | 0 or 1 |  |  |
| 2D detailed drawings | 0 or 1 |  |  |
| Electrical/electronic wiring diagrams | 0 or 1 |  |  |
| Updated manufacturing schedule | 0 or 1 |  |  |
| Operation manual | 0 or 1 |  |  |
| Maintenance & repair manual | 0 or 1 |  |  |
| Printed and presented in folder | 0 to 1 |  |  |
| Possible marks | 0 to 7 |  |  |

| Total possible marks for portfolio (your mark out of possible 17 marks) |  |
| --- | --- |

NOTE: Your total mark for the portfolio will be modified to a maximum mark of 10%, i.e. your mark x 10/17 which will be your final portfolio mark.

Judges Note:

When assessing the portfolio using the above criteria, it is sufficient to award marks for inclusion of the information when it comes up to the minimum requirements, rather than consider the actual detail contained within it.

Marks are awarded for each item as if acceptable – full marks or if not acceptable - zero marks (there will be no graduated marks)

The minimum requirements will be decided by the experts prior to the competition.

11. Marking scheme for Design & Production costs

Tariff cost calculation - all costs will be calculated in Euros (€’s)

Working hours: 					€20.00 /person/hour.

NOTE: When one member of the team is working, all team members will be costed therefore team cost would be €60.00 /hour

Additional price for machining:

For a conventional mill and lathe:				€15.00/hour

For CNC machining  						€25.00/hour

Welding, computer, grinding, sheet metalwork,

drilling, saw							€10.00/hour

Consultant or training						€40.00/hour

The minimum booking time for each machine is fifteen (15) minutes.

Raw material:

Each team will provide evidence of raw material costs (without discounts applied)

Purchased items:

For each used purchased item, the team has to hand in a receipt. (or current catalogue prices without discounts)

NOTE: The overall production and build costs of the vehicle will be recorded and the total costs will be calculated based upon the above costs. Cost penalties will be applied if project does not meet specification.

The cheapest completed vehicle after cost penalties have been applied, that best meets the specification will win the vehicle component of the competition

The conversion of build cost to a final points total will be done in line with the world skills spreadsheet and conversion formula.

Note:

No maintenance will be permitted on the vehicle after the competition and during the evaluation process.

12. Marking scheme for practical operations

a) Performance Evaluation

“The vehicle will have both speed and braking controls and be designed to achieve maximum speed of 6Km/h, climb an incline of 11,3 degrees (1 in 5) ten times in an hour with same battery charge and has to do controlled declines. It must be capable of transporting one person up to 75Kg and have capacity to carry 5Kg of luggage attached to the vehicle, within a maximum load space of 20cm x 20cm x 15cm.  The overall turning circle must be within a 2.0 metre diameter.”

NOTE: vehicle will be tested with a payload of 80kgs and all performance evaluations must be able to be completed without recharging the battery.

No physical help from the driver/rider either to assist start or to assist stop the vehicle is permitted except using the speed and braking controls.

When testing the vehicle, the driver/rider must wear a helmet. Dynamic performance tests will not be permitted if helmet is not worn.

Any team member may be the nominated test driver/rider but all up test weight must be at least 80kgs.

Charging the battery between tests will not be permitted. All performance tests must be conducted with the one battery charge.

Penalty: based on Percentage of total build cost

- The battery is capable of a range of 8km (with same battery charge)

- 15% cost penalty for each kilometre less than 8km the vehicle is unable to complete (max penalty 80%)

Evaluation procedure:

Vehicle will required to complete a distance of 8km. Driver/rider may be changed but must not have a payload less than 80kgs at any time during testing. A backpack with extra weights may need to be used so that the payload is 80kgs. Forward movement will not be permitted by pushing with driver’s feet. A maximum of 8 stops may be made during this test (emergency stops excluded). Any stop will not be more than 1 minute.

- The battery is capable of sustaining ten (10) slope incline and decline tests within one hour (with same battery charge)

- 5% cost penalty for each test less than 10 tests vehicle is unable to complete (max penalty 50%)

Evaluation procedure:

Incline test - vehicle will be stationary and positioned at bottom of incline, and must be able to complete 10 incline tests.

Decline test – vehicle will be stationary and positioned at top of incline, and must be able to complete 10 decline tests.

- The vehicle will achieve a speed of 6 kph

- 5% cost penalty for each kph below (max penalty 30%)

Evaluation procedure:

Vehicle will be timed over a set distance, three (3) times and an average speed calculated. A flying start is permitted.

- The vehicle will have adjustable speed controls

- 6% cost penalty if not

Evaluation procedure:

The vehicle must have some adjustable speed controls and using these controls must be able to demonstrate at least three (3) different moving speeds.

- The vehicle will have braking controls

- 6% cost penalty if not

Evaluation procedure:

The vehicle must have some braking controls and using these controls must be able to stop in a distance of 2 metres from a speed of 3kph.

- The vehicle will climb an incline of 11.3 degree (1 in 5)

- 10% cost penalty if unable

Evaluation procedure:

The vehicle when tested with the incline test above, completes at least one incline it will be considered as being able to climb that incline.

- The vehicle will traverse down the incline of 11.3 degree (1 in 5) with a controlled decline

- 10% cost penalty if unable

Evaluation procedure:

The vehicle when tested with the decline test above, must make three (3) complete stops in at least one (1) test run. If able to complete this, it will be considered as being able to have a controlled decline.

- The luggage load space is 20cm x 20cm x 15cm

- 3% cost penalty if a box this size is unable to fit

Evaluation procedure:

A rectangular box 20cm x 20cm x 15 cm must be able to be fastened securely to the vehicle.

- The vehicle will turn 180o within a 2 metre diameter circle

- 2% cost penalty for each 100mm over 2 metres (max penalty 10%)

Evaluation procedure:

A 2 metre diameter circle will be drawn on the floor. The vehicle must approach the circle, turn at least 180o and then exit the circle. At no time during this test is any part of the driver/rider permitted to touch the floor. Each team will be permitted two attempts to achieve this test and best result will be recorded.

Note: vehicles will be tested on a ramp inclined at 11.3o (1 in 5). The ramp length will be 5m long

Maximum penalty applied for this section could be 205% of build cost

b) Battery and Charge Systems

“The vehicle will operate by battery and must be designed to re-charge within a typical working day of 8 Hours. However, it should also be capable of fast charging within 2 hours. Evidence of initial battery conditions and charging indication must be provided.”

“The Vehicle must incorporate systems to cover night-time use; these must be visible from both the rear and front of the vehicle and provide visible illumination from a distance of 50m during dusk and darkness hours. It will also include a method of isolating the power when not in operation. An audible and visual warning will be required when the battery power runs low. The operational position should be adjustable to cover a range of differing height positions.”

- The battery is capable of recharging within 8 hours

- 5% cost penalty if not able to recharge in 8 hours

Evaluation procedure:

Difficulty with assessing this so suggest that this evaluation not be considered. There must be a charging circuit see item below

- The battery is capable of a fast charge within 2 hours

- 5% cost penalty if not able to fast charge within 2 hours

Evaluation procedure:

Difficulty with assessing this so suggest that this evaluation not be considered. There must be a charging circuit see item below

- There is a charging circuit

- 20% cost penalty if no charging circuit

Evaluation procedure:

Battery voltage will be checked before and during the battery charging to check if there is a charging circuit.

- There is a visible charging indication

- 5% cost penalty if not visible

Evaluation procedure:

There must be an indicator to indicate the battery is being charged.

- There is a visible indication of battery condition

- 5% cost penalty if not visible

Evaluation procedure:

There must be an indicator to indicate the charged level of the battery.

- There is a power isolation

- 5% cost penalty if no isolation

Evaluation procedure:

There must be a switch to isolate battery power, with an indicator label to identify whether the isolation is on or off

- There is an audible/visible warning of low battery

- 5% cost penalty if no warning indication

Evaluation procedure:

When battery level is low!! an audible/visible warning should be heard or seen

- Lights visible from rear of vehicle

- 5% cost penalty if no visible lights

Evaluation procedure:

There must be light/s visible at rear of vehicle and be capable of being switched on/off

- Clear light/s at front of vehicle to be capable of illuminating the roadway in front of vehicle

- 5% cost penalty if no lights

Evaluation procedure:

There must be clear light/s at front of vehicle and be capable of being switched on/off

Maximum penalty for this section is 50% of build cost

c) Weight

“The complete unit should be light and portable”

Evaluation Process

Each unit packed in transportation enclosure, complete with maintenance tools will be weighed.

- For each kilogram (or part thereof) in excess of the lightest complete unit 5% cost penalty (limited to 25% maximum)

Evaluation procedure:

The vehicle complete with guards and transportation enclosure, maintenance tools, battery charger, repair and maintenance manual and operation manual will be weighed. If all components are not presented for weighing the maximum penalty of 25% will be applied.

Maximum cost penalty for this section 25% of build cost

d) Assembly

“The vehicle should be readily assembled within a target time of 2 minutes and transported within an enclosure no bigger than 1m x 0.3m x 0.3m.”

“The operational position should be adjustable to cover a range of differing height positions”

- Assembly time for vehicle is less than 2 minutes

- 10% cost penalty for each minute over the 2 minutes time

(penalty limited to 30% maximum)

Evaluation procedure:

The vehicle is to be placed in the transportation enclosure with all maintenance tools, battery charger, and manuals a distance of 1 metre away from team. The enclosure must be closed and latched. The vehicle is to be removed from the enclosure and assembled into its normal operating configuration using only the maintenance tools supplied from the transportation enclosure. The vehicle must then be operated for 20 metres.

The assembly time will be timed.

All three (3) members of the team can be involved in the assembly of the vehicle.

- The vehicle does not fit in the specified envelope.

-5% cost penalty

Evaluation procedure:

The vehicle is to be placed in the transportation enclosure (1m x 0.3m x 0.3m) with all maintenance tools, battery charger and manuals

All three (3) members of the team can be involved in the placing the vehicle into the enclosure.

- The operating height is adjustable

- 5% cost penalty

Evaluation procedure:

The vehicle must have a height adjustment of at least 100mm on steering control and seating (if applicable)

Only the maintenance tools supplied from the transportation enclosure may be used to make this adjustment

Maximum cost penalty for this section 40% of build cost

e) Transportation Enclosure

“The enclosure of the unit has to be able to withstand handling forces of 500 N. The materials and components used throughout should be suitable for the purpose and the environment in which they will be installed.”

“Any tooling for repair and/or maintenance should be contained and fixed within the enclosure.”

Note: Manufacture of the transportation enclosure can be done prior to the competition.

The enclosure test will be carried out as first performance test, and if the vehicle is damaged no maintenance will be allowed. This may have an effect on further testing of your vehicle.

- Applied force – if enclosure is damaged during testing and affects the unit inside

- 10% cost penalty

Evaluation process: This test will be carried out as the first performance test. The enclosure will be tested by applying a static force of 500N from a 50mm diameter test bar at 3 locations on the enclosure. It may deflect, but should not damage the vehicle or any equipment within the enclosure. Note if your vehicle is damaged during this test it could have an effect on further testing of the vehicle.

- Sharp edges

- 1% cost penalty for each sharp edge found (max 5%)

Evaluation process: Check for any sharp edge/s on inside and outside of transport enclosure.

Maximum cost penalty for this section 15% of build cost

f) Vehicle Safety Requirements

“The Vehicle should be easily maintained and/or repaired with quick-fit or plug-in units with minimum skill and tooling requirements”

- Trap points

- 1% cost penalty for each trap point (max 5%)

Evaluation process: Check for any trap or pinch points that could trap a 12mm diameter rod x 60mm long, while vehicle is in normal operating configuration. Outside diameter of the wheels making contact with the floor will not be considered a trap point.

- Exposed Electrical connections

- 2% cost penalty for each exposed area (max 10%)

Evaluation process: Check if any exposed electrical connection has any exposed wire not fitted with insulation or a suitable covering

- Access to moving parts (wheels to be guarded by a sweep angle of not less than 180o)

- 10% cost penalty if not adequately guarded

Evaluation process: Check if any drives can be accessed by a 12mm diameter rod x 60mm long while vehicle is in normal operating configuration. Wheels must be guarded to a sweep angle not less than 180o, therefore access to 180o sweep angle of wheel/s is permitted. Contact with the portion of wheel that is unguarded is acceptable

Maximum cost penalty for this section 25% of build cost

g) Maintenance and Repair

“The Vehicle should be easily maintained and/or repaired with quick-fit or plug-in units with minimum skill and tooling requirements. Any tooling for repair and/or maintenance should be contained and fixed within the enclosure.”

Evaluation process:- The following tests will be performed individually and the total time calculated, only tools specified in the maintenance manual and contained in the enclosure or attached to the vehicle will be allowed.

1. The battery/batteries is to be completely removed from the vehicle in its normal operating configuration, placed statically 1 metre away and then replaced and connected. If multiple batteries are used all the batteries are to be removed but may remain connected to each other. The vehicle must then be operated for 20 metres.

2. The drive unit/motor to be completely removed from the vehicle placed statically 1 metre away, and then replaced and connected. The drive sprocket, pulley or coupling may still be mounted on the motor shaft. The vehicle must then be operated for 20 metres.

Note: where the vehicle has multiple drive motors only one motor is required to be removed. The experts will nominate which drive motor is required to be removed.

3. A drive wheel must be completely removed from the vehicle placed statically 1 metre away and then fully replaced and connected. The vehicle must then be operated for 20 metres. The drive wheel can have the axle and the drive sprocket or pulley still assembled but must remove any chains or belts connected to the wheel.

Note: where the vehicle has multiple driving wheels only one driving wheel is required to be removed. The experts will nominate which drive wheel is required to be removed.

Penalty:

- For each minute (or part thereof) in excess of the fastest total time

-3% cost penalty (limited to 25% maximum)

Evaluation process: See test process  above

- If tools are not fixed within the enclosure

  - 4% cost penalty

Evaluation process: All tools specified in the maintenance manual must be fixed securely within the enclosure

- Additional tools required to complete test

- 2% penalty for each additional tool required (max 6%)

Evaluation process: For each additional tool required to attempt the maintenance test above, penalty will apply.

Maximum cost penalty for this section is 35% of build cost

h) Documentation

“Compile a dossier of the complete process.”

Drawings

From MTC technical Description

1.4.1 	Design - Ability to interpret, construct, and modify engineering CAD drawings, to work with 3D modelling, and to convert both to CAM. Use CAM to generate toolpaths using a suitable postprocessor.

Drawing standard will be ISO-A or ISO-E depending on country involved.

NOTE: All drawings to be created in electronic format and paper copies included in Section B of the portfolio

- 3D Assembly drawing

- 10% cost penalty if not acceptable

Evaluation process: The drawing should be at least 90% complete and correct and:

    - have title block with:

    - Name

    - Scale

    - Drawing number

    - Projection system

    - border

    - have parts/material list

    - parts/material list to be referenced to drawing

- 2D detail drawings

- 2% cost penalty for each manufactured component without an acceptable detail drawing (max cost penalty – 10%)

Evaluation process: The drawing should be at least 90% complete and correct and:

    - have title block with:

    - Name

    - Scale

    - Drawing number

    - Projection system

    - material

    - drawing border

    - have appropriate views

    - dimensioned and toleranced with surface finish symbols where appropriate

    - sections (if required)

    - notes (if required)

    - guards are not required to be drawn

- Electronic/electrical wiring circuit

- 5% cost penalty for each circuit without an acceptable drawing

(max penalty – 10%)

Evaluation process: The drawing should be at least 90% complete and correct and:

    - have title block with:

    - Name

    - Drawing number

    - drawing border

    - standard electrical/electronic symbols with component values

Judges Note:

When assessing the drawings using the above criteria, it is sufficient to award marks for the information and interpretation when it comes up to the minimum requirements, rather than consider the actual detail contained within it.

i) Manuals

From MTC technical Description

1.3.8 	Documentation - Document the process including header page, index, and descriptive overview of the task, hard copy of any programs, instructions for setting up and assembly, and any relevant drawings.

This is to be presented in electronic word processed format and could include text, sketches, diagrams and photos. A paper copy of this manual is to be printed and included in Section B of the portfolio document.

- Operation manual

- 10% cost penalty if not acceptable

Evaluation process: The operation manual will be evaluated by a team of three (3) experts and should consist of:

- Index/table of contents with page numbers

- List of items supplied with the vehicle, including the transportation enclosure, chargers, maintenance tools and manuals

- Function and operation of all controls/switches including brakes, speed control, battery condition, power isolation, lights etc.

- General specification and performance of vehicle, including weight, maximum speed, weight capacity, turning circle etc.

- Safety considerations when using the vehicle

- General hints on using vehicle

- Maintenance & repair manual

- 10% cost penalty if not acceptable

Evaluation process: The maintenance and repair manual will be evaluated by a team of three (3) experts and should consist of:

- Index/table of contents with page numbers

- List of maintenance tools supplied

- Instructions to include

- assembly and packing vehicle into transportation enclosure

- replacement of battery/batteries

- charging of battery/batteries

- replacement of drive motor/s

- replacement of drive wheel/s

- replacement non driven wheels

- Lubrication of vehicle (if required)

j) Manufacturing Schedule

The original manufacturing plan will need to be modified after the surprise project is known to each team and must reflect the manufacturing process undertaken by each team.

This manufacturing plan is to be done in electronic format with paper copy included in Section B of portfolio document

- manufacturing schedule to include manufacturing processes of the surprise project

- 10% cost penalty if not acceptable

Evaluation process: The manufacturing schedule manual will be evaluated by a team of three (3) experts and must have the manufacture of the surprise project included.

Maximum cost penalty for this section is 60% of build cost

Note: The maximum cost penalty that could be applied is 455% of raw build cost.